So2rv provides a way to compute geocentric satellite orbits from two observations on a single satellite pass.  Two observations are sufficient to compute a circular orbit which will give fairly good information on the orbit inclination, ascending node, mean anomaly, and mean motion.  Eccentricity is zero for a circular orbit and the perigee is meaningless.  The orbit provides a good approximation from which to begin higher order corrections.

Three matrices are required as input.  [OD] is the matrix that holds the observation data.  Each row is an observation in the form:

Julian date, right ascension(degrees), declination(degrees)

[OD] is only used to pass the Julian date for each observation to the program.  The line of sight data is reduced from [OD] outside the program and passed inside the matrix [LL].  Each row in [LL] holds unit vectors in the equatorial coordinate system pointing from the observer positions to the corresponding satellite positions at each observation time.

[RD] is the matrix that holds the geocentric vectors to the observers position at each of the observation times.  The units are earth radii in the equatorial coordinate system.

The program assumes an initial satellite distance from the geocenter of rr = 1.1 earth radii.  This is a good approximation for most visual satellite observations.

The loop starts here.  Given a length for the satellite position vectors, the program can calculate the direction of the position vectors for each of the two observations, rr1 and rr2, by calling So2r.  When these two vectors are known we can calculate the angle, theta, between them using the dot product formula:

theta = acos(dot(rr1, rr2) / (rr*rr))

Since the time between the observations, delt, is also known there is but one velocity that will carry a satellite the circular distance at radius rr through an angle of theta in time delt.  This velocity, vv, in distance units/time units is:

vv = theta * rr * tu / delt

The constant tu converts delt from time units of days into time units such that a circular orbit at rr = 1 would have a velocity of vv = 1.  But this velocity, vv, is not necessarily the circular orbit velocity.  vv is a function of theta, which is a function of our assumed satellite orbit radius, rr.  If rr increases then vv increases in this calculation because of the longer arc. Orbit dynamics however requires satellites to have a lower velocity at larger values of rr.

We can now calculate the circular orbit radius that would have the computed velocity vv:

rro = 1 / (vv*vv)

If our guess radius, rr, was too big then vv was too fast and we will find that rro < rr.  If our guess for rr was too small then rro > rr.  The correction, rro, will always be in the right direction but its magnitude is too large to converge.  A little weighted averaging (100:1) will damp the divergent behavior:

rr = .01 * (99 * rr + rro)

Continue the loop until the desired accuracy is achieved.  Once the satellite orbit radius converges, we are in a position to calculate the orbit.  At this point we will have the satellite position vector, rr2, at epoch od[1][0], and the magnitude of the velocity vector, vv.  We only need the direction of the velocity vector to determine the orbit.

The direction of the velocity vector may be found from the cross product of the two position vectors, rr1 and rr2.

(rr1 x rr2) x rr2 / norm((rr1 x rr2) x rr2)

gives a unit vector perpendicular to rr2 and in the desired direction.  We multiply this unit vector by vv to get the velocity vector at rr2 and we are done.